4. TESTING

Now we find out the complexity of the module to determine the possible independent paths and hence develop test cases.

PSEUDOCODES

4.1 NodeMCU-MQTT

START

SET wifi_ssid to "Ravi188"(WiFi connection Name)						[1]

SET wifi_password to "115204416"(WiFi Password)						[2]

SET mqtt_server to "test.uniconvergetech.in"						[3]

SET mqtt_user to "guest"									[4]

SET mqtt_password to "guest"								[5]

SET temperature_topic to "sensor/temperature"						[6]

SET humidity_topic to "sensor/humidity"							[7]

declare a 100 characters array								[8]

initialize lastMsg to 0										[9]

initialize lastRecu to 0										[10]

initialize debug to FALSE									[11]

SET DHTPIN to D4										[12]

SET DHTTYPE to DHT11									[13]

declare dht and configure DHTPIN of DHTTYPE						[14]

declare a WiFiClient espClient								[15]

declare a PubSubClient client with a WiFiClient						[16]

Function setup

Pass In:nothing

Start a serial communication from nodeMCU at a baud rate of 9600		[17]

configure D2 pin as OUTPUT pin							[18]

call setup_wifi() to setup WiFi connection						[19]

configure a MQTT connection with mqtt_server on port 1883			[20]

set a handling function callback() to be executed when a MQTT message is received[21]

Pass Out:nothing

Endfunction

Function setup_wifi

Pass In:nothing

take a delay of 10ms									[22]

print "Connecting to (wifi_ssid)"							[23]

establish a WiFi connection using wifi_ssid & wifi_password			[24]

REPEAT

take a delay of 500ms								[25]

print "."									[26]

UNTIL WiFi connection doesn't get established					[27]

print "WiFi OK => ESP8266 IP Address: "						[28]

print local IP address									[29]

Pass Out:nothing

Endfunction

Function reconnect

Pass In:nothing

REPEAT

print "connecting to mqtt broker..."						[30]

IF MQTT connection get established 						[31]

THEN print "OK"							[32]

ELSE

print Client's state as error						[33]

print "Wait 5 seconds before to retry"				[34]

take a delay of 5000ms						[35]

ENDIF

UNTIL client connection with server doesn't get established			[36]

Pass Out:nothing

Endfunction

Function loop

Pass In:nothing

IF client's MQTT connection hasn't been established				[37]

THEN call reconnect()									[38]

ENDIF

initialize 'now' with the milliseconds passed up till now				[39]

IF lastMsg timing was a minute ago THEN						[40]

SET lastMsg to now								[41]

read Humidity from DHT sensor to h						[42]

read temperature from DHT sensor to t					[43]

IF read values are not numerical THEN					[44]

print "Please change DHT sensor!"					[45]

return out of function							[46]

ENDIF

IF debug sets TRUE THEN							[47]

print "Temperature : t"						[48]

print "Humidity : h"							[49]

ENDIF

publish temperature & humidity on their respective topics created on node-red[50]

ENDIF

IF lastRecu timing was more than a minute ago THEN				[51]

SET lastRecu to now								[52]

subscribe to the topic led/switch1 for controlling device			[53]

ENDIF

Pass Out:nothing

Endfunction

Function callback

Pass In: Topic, payload in bytes, length of msg

initialize counter i to 0								[54]

IF debug is true THEN									[55]

print "Message recu=> topic: " + topic					[56]

print "| lonueur: " + length in string form					[57]

ENDIF

FOR every character of msg								[58]

copy from payload into message_buff					[59]

ENDFOR

add a null character at end of message_buff						[60]

convert message_buff into a string to msgString					[61]

IF debug is true THEN									[62]

print "Payload: " + msgString							[63]

ENDIF

IF msgString is 'ON' THEN								[64]

set D2 Pin at LOW state							[65]

ELSE

set D2 pin at HIGH state							[66]

ENDIF

Pass Out:nothing

Endfunction

It’s cyclomatic complexity is:

Regions = 11

Predicate Nodes+1: 10+1 = 11

Edges-Vertices+2: 75-66+2 = 11

Independent Paths:

- 1-37-39-40-51-54-55-58-62-64

- 1-37-39-40-51-54-55-58-62-63-64

- 1-37-39-40-51-54-55-56-57-58-64

- 1-37-39-40-51-52-53-54-55-58-64

- 1-37-39-40-44-47-50-51-54-55-58-64

- 1-37-39-40-44-47-48-49-50-51-54-55-58-64

- 1-37-39-40-44-45-46-47-48-49-50-51-54-55-58-64

- 1-37-38-30-31-32-36-39-40-51-54-55-58-64

- 1-37-38-30-31-33-34-35-36-39-40-51-54-55-58-64

4.2 TEST CASES

TEST CASE ID-01

| SECTION-I (Before Execution) | SECTION-II (After Execution) |
| --- | --- |
| PURPOSE: Checking the functionality of system | EXECUTION HISTORY:- |
| PRE-CONDITIONS: Person going to operate must be member of that home where system is installed | RESULT: Fails, No connected message displayed |
| INPUTS: Passing power to the circuit using USB cable | IF FAILS, ANY POSSIBLE REASON(OPTIONAL): We must have tried to connect PC and NodeMCU device on different Wi-Fi networks |
| EXPECTED OUTPUTS: Wi-Fi connection setup message on Serial Monitor | ANY OTHER OBSERVATION: None |
| POST-CONDITIONS: None | ANY SUGGESTION: Always Connect both devices on same network |
| WRITTEN BY: Saumya | RUN BY: Haripriya |
| DATE: 19 April 2021 | DATE: 19 April 2021 |

TEST CASE ID-02

| SECTION-I (Before Execution) | SECTION-II (After Execution) |
| --- | --- |
| PURPOSE: Checking the functionality of system | EXECUTION HISTORY:- |
| PRE-CONDITIONS: Person going to operate must be member of that home where system is installed | RESULT: Wi-Fi connection setup successfully |
| INPUTS: Passing power to the circuit using USB cable | IF FAILS, ANY POSSIBLE REASON(OPTIONAL): |
| EXPECTED OUTPUTS: Wi-Fi connection setup message on Serial Monitor | ANY OTHER OBSERVATION: |
| POST-CONDITIONS: None | ANY SUGGESTION: |
| WRITTEN BY: Saumya | RUN BY: Muskaan Talat |
| DATE: 20 April 2021 | DATE: 20 April 2021 |

TEST CASE ID-03

| SECTION-I (Before Execution) | SECTION-II (After Execution) |
| --- | --- |
| PURPOSE: Checking the functionality of system | EXECUTION HISTORY:- |
| PRE-CONDITIONS: Person going to operate must be member of that home where system is installed | RESULT: Connected to MQTT broker after getting setup with W-Fi connection successfully |
| INPUTS: Passing power to the circuit using USB cable | IF FAILS, ANY POSSIBLE REASON(OPTIONAL): |
| EXPECTED OUTPUTS: Client Connection to MQTT broker on port 1883 and display message on Serial Monitor | ANY OTHER OBSERVATION: Node-Red MQTT nodes represents setup connection |
| POST-CONDITIONS: None | ANY SUGGESTION: |
| WRITTEN BY: Saumya | RUN BY: Muskaan Talat |
| DATE: 20 April 2021 | DATE: 20 April 2021 |

TEST CASE ID-04

| SECTION-I (Before Execution) | SECTION-II (After Execution) |
| --- | --- |
| PURPOSE: Checking the functionality of system | EXECUTION HISTORY:- |
| PRE-CONDITIONS: Person going to operate must be member of that home where system is installed | RESULT: Dashboard Screen with expected items |
| INPUTS: Deploy the circuit on Node-Red & open Dashboard | IF FAILS, ANY POSSIBLE REASON(OPTIONAL): Circuit may not be successfully deployed |
| EXPECTED OUTPUTS: 2 Gauge meters (of temperature and Humidity) + a toggle button displayed on dashboard | ANY OTHER OBSERVATION: |
| POST-CONDITIONS: None | ANY SUGGESTION: None |
| WRITTEN BY: Haripriya | RUN BY: Damini Patel |
| DATE: 21 April 2021 | DATE: 21 April 2021 |

TEST CASE ID-05

| SECTION-I (Before Execution) | SECTION-II (After Execution) |
| --- | --- |
| PURPOSE: Checking the functionality of system | EXECUTION HISTORY:- |
| PRE-CONDITIONS: Person going to operate must be member of that home where system is installed | RESULT: LED turned ON |
| INPUTS: Turn ON toggle button dashboard | IF FAILS, ANY POSSIBLE REASON(OPTIONAL): Loose Connections in circuit may result in failure |
| EXPECTED OUTPUTS: Home LED gets turned on | ANY OTHER OBSERVATION: Circuit gets heated up after some time. So, it’s recommended not to keep lights on for a very long duration. |
| POST-CONDITIONS: None | ANY SUGGESTION: |
| WRITTEN BY: Haripriya | RUN BY: Muskaan Talat |
| DATE: 22 April 2021 | DATE: 22 April 2021 |

TEST CASE ID-06

| SECTION-I (Before Execution) | SECTION-II (After Execution) |
| --- | --- |
| PURPOSE: Checking the functionality of system | EXECUTION HISTORY:- |
| PRE-CONDITIONS: Person going to operate must be member of that home where system is installed | RESULT: LED turned OFF |
| INPUTS: Turn OFF toggle button dashboard | IF FAILS, ANY POSSIBLE REASON(OPTIONAL): Wrong connections |
| EXPECTED OUTPUTS: Home LED gets turned off | ANY OTHER OBSERVATION: |
| POST-CONDITIONS: None | ANY SUGGESTION: |
| WRITTEN BY: Haripriya | RUN BY: Damini Patel |
| DATE: 22 April 2021 | DATE: 22 April 2021 |